Please add a Canopen Node in systemmanager

- aktiveren General Can-Node (direkt access to layer 2, No NMT)

- choice a randrom Node Id ( it is a virtulle slave just for sending CAN commando)

- add two variable unter RXPDO1

- change COB ID to 000 ( if you want to send NMT Message)

- links the variabe of RXPDO with your PLC Program.

- Rum PLC Porgram, ( if Variable20 =0x81, Variabe21=07, it means Node 7 will be reset)